Below is a summary of the methods used to determine the carbon footprints, quantified by Global Warming Potential (GWP), for direct gas emissions and chemicals/process gases used in fabrication.

- Direct emission of gases during fabrication

For gases that are generated and emitted into the atmosphere during fabrication, we estimated their impacts using the GWP characterization factors (in kg CO2-eq per kg gas) provided by the TRACI 2.1 method.

TRACI (Tool for the Reduction and Assessment of Chemical and Other Environmental Impacts) is a life cycle impact assessment (LCIA) method developed by the U.S. Environmental Protection Agency1. It includes 10 environmental impact categories, among which GWP represents global warming impacts over a 100-year time horizon.

- Chemicals and process gases

The carbon footprints of chemicals and process gases used during fabrication were quantified through process-based life cycle assessment (LCA) modeling, with impacts characterized using the TRACI 2.1 method. The main modeling steps are summarized below.

Functional unit

All material- and gas-related impacts were modeled and calculated per 1 kg of substance (e.g., 1 kg of process gas or chemical input). Using a 1 kg functional unit ensures consistent comparison across different materials used in semiconductor fabrication.

System boundary

The system boundary is cradle-to-gate, including raw material extraction, product manufacturing, and transportation. Device use-phase and end-of-life stages are excluded.

Life cycle inventory (LCI) analysis

Ecoinvent 3.9 was used as the background database2. For chemicals and process gases available in Ecoinvent database, we directly adopted the corresponding datasets. For materials not available in Ecoinvent database but whose synthesis routes are described in published literature, we conducted process modeling based on reported preparation methods, collecting input/output information to construct foreground inventory data. For materials where detailed industrial synthesis data are not available, we applied a stoichiometric approach to estimate the foreground LCI based on chemical reactions. All foreground data and associated background processes were compiled to generate an aggregated LCI for each chemical or process gas.

Life cycle impact assessment (LCIA)

Based on the compiled LCI data, all greenhouse gas emissions were quantified in terms of GWP using the TRACI 2.1 method, which is widely used in North American LCA studies. We selected TRACI to ensure methodological consistency with U.S. environmental assessment practices and to support comparability across materials, gases, and process steps within a U.S. context.

Interpretation notes

The reported GWP values are derived from multiple approaches, including LCIA characterization factors for direct gas emissions and process modeling based on standardized LCA procedures and documented assumptions, which are not direct facility measurements.

1.	Bare, J. C. Tool for the Reduction and Assessment of Chemical and Other Environmental Impacts (TRACI), Version 2.1 - User’s Manual; EPA/600/R-12/554 2012.

2.	Wernet, G. et al. The ecoinvent database version 3 (part I): overview and methodology. Int J Life Cycle Assess 21, 1218–1230 (2016).